Krusty

KRUSTY — which stands for Kilopower Reactor Using Stirling Technology — a prototype fission power system developed by NASA and U.S. national laboratories to prove that small nuclear reactors can safely generate electricity for surface missions on the Moon or Mars.

It’s one of the most important real-world demonstrations of space nuclear power ever done.

Let’s walk through what it was, how it worked, and why it matters for companies like Radiant thinking about space.

⸻

⚛️ What Was KRUSTY?

KRUSTY was not a flight reactor.

It was a full nuclear ground test run in 2017–2018 to validate the Kilopower concept: a compact, uranium-fueled reactor producing ~1–10 kilowatts of electric power continuously for planetary surface operations.

The test campaign showed:

✔ the reactor could start safely

✔ operate steadily

✔ follow load changes

✔ shut down predictably

✔ and survive fault scenarios

That was a huge deal — it moved space reactors from paper to hardware.

⸻

🔧 How KRUSTY Worked

KRUSTY’s architecture was deliberately simple and ultra-robust.

Fuel & Core

•	Solid uranium-molybdenum alloy fuel

•	Fast-spectrum reactor

•	Heat-pipe cooled (no pumps)

•	Passive physics-based stability

Heat Transport

•	Sodium heat pipes carried heat from the core outward

•	No moving parts inside the reactor

Power Conversion

•	Stirling engines converted heat into electricity

•	High efficiency at small scale

•	Long-life mechanical systems

Control

•	Single control rod for startup and regulation

•	Strong negative temperature feedback (gets safer as it heats)

⸻

🌙 What Missions It Was For

NASA built Kilopower / KRUSTY specifically for:

•	Lunar surface bases

•	Mars habitats

•	ISRU plants

•	Science stations in permanently shadowed craters

•	Backup power for crewed missions

Think: 14-day lunar night solved.

⸻

🧪 Why KRUSTY Mattered

Before KRUSTY, space nuclear reactors were mostly theoretical in the U.S.

KRUSTY proved:

• small reactors can be built cheaply

• passive cooling works

• heat pipes survive nuclear conditions

• autonomous operation is feasible

• kilowatt-class space reactors are practical

It was the bridge between 1960s SNAP-era reactors and modern lunar/Mars power systems.

⸻

📉 Why It Didn’t Immediately Fly

Important nuance: KRUSTY succeeded technically, but:

•	It wasn’t a flight-qualified system

•	It didn’t go through launch approval

•	NASA didn’t yet have a funded lunar reactor mission

•	Budgets shifted toward other Artemis priorities

•	Higher-power systems (tens to hundreds of kW) became the next target

So it became a reference architecture, not an operational program.

⸻

🚀 How This Relates to Radiant & Today’s Space Reactors

For a company like Radiant Nuclear, KRUSTY is basically the proof-of-concept ancestor:

KRUSTY showed:

•	heat-pipe cooling works

•	passive safety is king

•	simple cores beat complex ones

•	autonomy is mandatory

•	launch-safe configurations are doable

Radiant-style systems would:

•	scale power upward (10s–100s of kW)

•	optimize for manufacturability

•	integrate modern autonomy

•	pursue actual flight missions

•	be wrapped in DoD/NASA acquisition pipelines

NASA’s Fission Surface Power (FSP) Program — What It Is

FSP is NASA’s initiative (in partnership with the U.S. Department of Energy (DOE)) to design, mature, and ultimately demonstrate a nuclear fission power system on the surface of the Moon — enough to support sustained operations for future lunar bases and inform future Mars missions.

•	This is not just technology research — NASA intends for it to become an operational system that could be deployed.

•	It picks up from Kilopower/KRUSTY’s success but expands to a higher power level and full system design.

⸻

⚡ What NASA Is Targeting Now

Power Level

•	Originally focused on a 40-kilowatt class system — enough to run infrastructure like habitats, ISRU systems, and science labs through lunar nights.

•	Recent industry engagement materials indicate an interest in ~100 kW systems to better support larger bases and dual-use civil/defense power architectures.

This is much bigger than KRUSTY (which was 1–10 kW), but still far below megawatt-class reactors.

⸻

🧠 How NASA Is Advancing FSP

1. Industry Feedback & Design Contracts

NASA has solicited industry input and concept designs to shape the system requirements and feed into future acquisition actions.

2. Collaboration with DOE

DOE (and national labs like Idaho National Lab) are co-leading the technology maturation and system design efforts, especially given DOE’s expertise in terrestrial reactors and advanced fuels.

3. Partnership Models

Recent guidance indicates NASA may use Space Act Agreements (similar to how commercial crew/cargo were done) where NASA buys power or service rather than the reactor hardware itself, potentially lowering upfront costs and integrating industry.

⸻

🗓️ When a Flight Might Happen

NASA and DOE publicly anticipate that a Fission Surface Power system could be ready to deploy to the lunar surface by the end of this decade (~2030) as part of Artemis and future lunar economy architecture.

U.S. leadership statements from NASA and DOE also specifically mention aiming for a lunar reactor within the next several years as part of broader space infrastructure goals.

⸻

📌 Why This Is a Big Deal

▶ Builds on KRUSTY

KRUSTY proved small fission reactors can run safely and stably. FSP aims to take that proof of physics and turn it into a flight-worthy, operational power system capable of supporting real lunar surface missions.

▶ Powers Long-Duration Bases

Solar alone struggles with lunar night (14 Earth days) and shadowed regions. FSP’s reactors would provide constant, abundant power regardless of sunlight, which is essential for long-term habitation and ISRU.

▶ Steps Toward Larger Systems

Demonstrating ~40–100 kW on the Moon lays the groundwork for even larger reactors in the future — potentially scaling toward megawatt-class capacity as needs grow.

⸻

🧠 One-Sentence Summary

NASA’s current space nuclear effort — the Fission Surface Power (FSP) program — is actively working with DOE and industry to develop and eventually deploy a ~40–100 kW class nuclear power reactor on the Moon by around 2030, moving beyond Kilopower to a practical, mission-ready system.